PRE-INSTALLATION VERIFICATION TEST OF HIGH STRENGTH BOLTS

Wisconsin Department of Transportation

DT2114          7/2013

This form should be filed with the completed project file as a permanent record.

Purpose of test: To perform and verify that tension obtained in three samples of each bolt lot is at least 5% above the minimum installation tension and record the results along with the fastener assembly lot information.

Instructions

- Choose a method to install bolts – calibrated torque wrench, turn of the nut method, or DTI washers.

- Fill in the information regarding each lot combination, noting that each combination to be used requires a separate test.

- Take 3 assemblies from the lot established in step 2 above.

- Install one assembly in a calibrated load cell and snug the bolt to ≈150 Ft/Lbs (for turn of the nut method) or 50% of required torque in Table 3, whichever is less(1,2).

- Match mark the nut, and the load cell plate.

- Turn the nut the required turn on Table 2.

- Record the tension for sample 1 and repeat for samples 2 and 3.

- Verify the readings are at or above the required tension in Table 1.

(1) If using a calibrated torque wrench to install bolts, bring each assembly to the tension required in Table 1 and record the torques and average to acquire the installation torque needed for installation.

(2) If DTI washers are being used, tighten until dimples are crushed according to manufactures guidelines compare the results and verify it meets the tension in Table 1. (Measure torque while nut is turning.)

| Project #: | Bolt | Nut | Washer |  | DTI (only if used) |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  |  |  |  |  |
| Lot Number |  |  |  |  |  |
| Diameter/Length |  | Bolt Grade A325 A490 |  | Note: Each combination of lots require a separate worksheet. |  |

| Pre-installation Verification Test | Sample 1 | Sample 2 | Sample 3 | Average if installing with torque wrench |
| --- | --- | --- | --- | --- |
| Compare minimum tension from Table 1 / Actual measured tension | / | / | / |  |
| Measured Torque – for torque wrench installation ONLY |  |  |  |  |

| Table 1 Minimum Installation Tension Required for the Pre-installation Test |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bolt Dia. | 3/4" | 7/8" | 1" | 1 1/8" | 1 1/4" | 1 3/8" | 1 1/2" |
| A325 Kips | 29 Kips | 41 Kips | 54 Kips | 59 Kips | 75 Kips | 89 Kips | 108 Kips |
| A490 Kips | 37 Kips | 51 Kips | 67 Kips | 84 Kips | 107 Kips | 127 Kips | 155 Kips |

| Table 2 Rotation Requirements for Turn of the Nut Method |  |
| --- | --- |
| Bolt length(L) relative to Bolt diameter(D) | Rotation |
| L≤4D | 1/3 Turn |
| L>4D/L≤8D | 1/2 Turn |
| L>8D/L<12D | 2/3 Turn |

This portion of the form is instruction for Inspection Testing

Instructions: Use 3 bolts from the lot established in step 3. Increase torque until reaching tension in Table 3, record torque and average the 3 torques – this is your inspection torque for this lot.

Note: 10% or a minimum of 2 bolts chosen randomly from each connection on the structure are to be tested with the established inspection torque.

| Inspection Torque for Inspection Testing ONLY | Sample 1 | Sample 2 | Sample 3 | Average Inspection Torque |
| --- | --- | --- | --- | --- |
| Measured Torque* at minimum tension in Table 3 |  |  |  |  |
| * Note: measure torque while nut is turning. |  |  |  |  |

| Table 3 Minimum Inspection Tension Required |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bolt Dia. | 3/4" | 7/8" | 1" | 1 1/8" | 1 1/4" | 1 3/8" | 1 1/2" |
| A325 Kips | 28 Kips | 39 Kips | 51 Kips | 56 Kips | 71 Kips | 85 Kips | 103 Kips |
| A490 Kips | 35 Kips | 49 Kips | 64 Kips | 80 Kips | 102 Kips | 121 Kips | 148 Kips |

Note: Table 3 is for Inspection Purposes. Table 1 is for the installation test. Table 1 has a 5% higher tension requirement.

| Verified By | Date (m/d/yyyy) |
| --- | --- |